Goal

Standardize LTMS across test types and provide a consistent template for all test types.  LTMS guidelines are suggested in order to provide the best possible basis for level-playing-field, unbiased, cost-effective candidate testing.

Issue 1:  Reference Oils

Reference Oils are requested and selected by the ASTM Surveillance Panel and Classification Panel.

Important guidelines for reference oil selection:

- Strive for reference oils that “mimic” candidate oil behavior.

- Reference oils should meet the chemical and physical limits of the category.

- Reference oils do not need to pass every parameter for the test, but they should be “around” the pass/fail limit.  Oils that are more than 2 standard deviations above or 1 standard deviation below any pass/fail parameters are not desirable.

Issue 2:  Test Targets

Test targets are developed by the Surveillance Panel.  However, given the importance of test targets to the success of LTMS, it is worth considering target development as a responsibility of the LTMS Task Force or ACC.

Important guidelines for target development:

- Strive for a homogeneous dataset to set targets.

- Outliers should not be removed from the target dataset unless special cause can be identified.  In other words, if outlier test results can be attributed to a set of parts, a certain lab practice, or a certain lab, then the outlier results may be removed or adjusted through statistical regression analysis..

- Target development dataset should be developed within as short a timeframe as possible.

- Time effect should be checked in every target development dataset.  If time is a factor, then it needs to be accounted for and corrected.

- Issues involving oil by lab interactions should be resolved before final targets are set.

- Mean and standard deviation targets should be based on data that is as close to normally (Gaussian) distributed as possible.  In order to achieve this, use of a data transformation may be necessary.  In addition, a transformation may also be necessary to eliminate any relationships between oil means and standard deviations.  The assumption of normality is very important to the appropriateness of the LTMS control charts.

- Test standard deviation estimates should be developed based on the process monitored.  For example, if the data permits a variance components analysis, an estimate of the within engine standard deviation should be used for monitoring test engines.  An estimate of the within lab (between stand/engine) standard deviation should be used for monitoring labs, and an estimate of the within industry (between labs) standard deviation should be used for monitoring Industry.

- A minimum of 10 tests should be used to set targets.  If this is not possible, the targets should be recalculated at 10 tests and 30 tests.

- If targets are being developed for a re-blend of an oil, the old targets should be used until there are 10 tests on the re-blend.  Any time effects should be incorporated into the re-blend targets.  It is worth considering using the old targets as a prior, and updating the prior with the re-blend data to set the re-blend targets.

- Keep in mind that targets are developed to enable LTMS to achieve the best possible basis for level-playing-field, unbiased, cost-effective candidate testing.

Issue 3:  Determine Major Source of Variability

For each test type, the major source of variability needs to be determined in order to set the appropriate calibration requirements and source of severity adjustments.  The major source of variability should be determined by the Surveillance Panel.

Major Source:  Lab

In this situation, test parts and lab practice are the major sources of variation for test results.  The actual test engine or test stand might have an impact, but the influence is less likely.  The test operation should be largely static (steady-state) as opposed to dynamic.  There is typically a rebuild after each test.

Reference intervals and frequency would be based on time/candidates in the lab as opposed to the stand, although requirements may be placed on the stand as well.  Precision requirements and severity adjustments would apply to the lab.

Major Source:  Lab/Stand or Lab/Engine

In this situation, test parts and lab practice are the major sources of variation for test results, but the variability of the stand and/or engine cannot be discounted.  In this situation, while there might be a rebuild after each test, the rebuild does not consist of all major parts.  The test operation might also have more of a dynamic component.

Reference intervals and frequency would be based on time/candidates in the stand or engine, although additional requirements may be placed on the lab as well.  Precision requirements would apply to the stand or the engine as well as the lab and severity adjustments would be on a lab basis.

Major Source:  Engine

In this situation, the specific test engine is the major sources of variation for test results.  This is typically a flush and run test and/or a test with a large dynamic component.

Reference intervals and frequency would be based on time/candidates in the engine, although additional requirements may be placed on the lab as well.  Precision requirements and severity adjustments would apply to the engine.

Issue 4:  New Calibration

For a system in which the major source of variation is considered to be the Lab, consider a 4 test, reference test requirement to calibrate the lab (at a minimum, more may be warranted), and a 1 test, reference test requirement to calibrate each stand.  Reduced K does not apply.

For all other system types (Lab/Stand, Lab/Engine, Stand/Engine, Stand, Engine), consider 4 reference tests to calibrate the first Stand or Engine in the lab, 3 reference tests to calibrate the second, and 2 reference tests to calibrate each new, subsequent stand or engine (at a minimum, more may be warranted).  Since the calibration requirement is on the stand/engine, reduced K should apply for each new stand in a lab that has completed at least 6 reference tests and does not currently have any precision alarms.  The reduced K requirement should be for severity only as the precision in the 1 test case is redundant.  The reduced K is set such that the probability of a successful calibration under the reduced K scenario is equivalent to two consecutive successful calibrations under the original K scenario.  A full table is provided in the ‘Reduced K Calculations’ and ‘Reduced K Table’ tabs in Appendix A.

The K value of reference test acceptance (K values are the control chart limits and represent the number of standard deviations from target) may be set based upon a fixed 5% false alarm error rate (a higher false alarm error rate may be warranted, for example, for tests in which the reference frequency is very low) for each critical test parameter.  Non-critical test parameters (the definition of non-critical is, at the time being, at the determination of the appropriate Surveillance Panel) may be adjusted to an overall 5% false alarm error rate (once again, a higher false alarm error rate may be warranted) based upon the number of independent test parameters in the test.  Principal components that

explain at least 90% of the test variation should be used to determine the number of independent parameters from which to set the overall 5% false alarm error rate for non-critical parameters only.  Until the principal components analysis can be performed, minimum of 10 test results to perform the analysis, the individual, critical parameter, false alarm error rate shall apply.  The attached table in the ‘False Alarm Error Rates’ tab (see Appendix A) lists the false alarm error rates as a function of K, the number of parameters, and whether or not the control chart has a one-sided or two-sided limit.

Issue 5:  Exceed EWMA Lab Precision Alarm

For a system with lab based severity adjustments, perform the following actions.

- Special K and Reduced K no longer apply for the parameter.

- There are to be no lab severity adjustments for being severe for the parameter with the precision issue.

- Lab severity adjustments would continue for being mild in the parameter.

- Reduce the reference interval for the next scheduled reference test (whether or not it is the same stand or engine) by three candidate tests, or the number of days equivalent to three candidate tests. This might be a percentage of the calibration period.

For a system with stand or engine based severity adjustments, perform the following actions.

- None, but notify TMC.

Issue 6:  Exceed EWMA Stand/Engine Precision Alarm

For a system with stand or engine based severity adjustments, perform the following actions.

- Special K and Reduced K no longer apply for the parameter.

- There are to be no stand/engine severity adjustments for being severe for the parameter with the precision issue.

- Severity adjustments would continue for being mild in the parameter.

- Reduce the reference interval for the next scheduled reference test in the offending stand/engine by three candidate tests, or the number of days equivalent to three candidate tests. This might be a percentage of the calibration period.

For a system with lab based severity adjustments, perform the following actions.

- Special K and Reduced K no longer apply for the parameter.

- Reduce the reference interval for the next scheduled reference test in the offending stand/engine by three candidate tests, or the number of days equivalent to three candidate tests.  This might be a percentage of the calibration period.

Issue 7:  Exceed Shewhart Lab Precision Warning Alarm

For a system with lab based severity adjustments, perform the following actions.

- Special K and Reduced K no longer apply for the parameter.

- Reduce the reference interval for the next scheduled reference test (whether or not it is the same stand or engine) by one candidate test, or the number of days equivalent to one candidate test.  This might be a percentage of the calibration period.

For a system with stand or engine based severity adjustments, perform the following actions.

- None, but notify TMC.

Issue 8:  Exceed Shewhart Lab Precision Action Alarm

For a system with lab based severity adjustments, perform the following actions.

- Special K and Reduced K no longer apply for the parameter.

- Reduce the reference interval for the next scheduled reference test (whether or not it is the same stand or engine) by four candidate tests, or the number of days equivalent to four candidate tests.  This might be a percentage of the calibration period.

For a system with stand or engine based severity adjustments, perform the following actions.

- None, but notify TMC.

Issue 9:  Exceed Shewhart Stand/Engine Precision Warning Alarm

For a system with stand or engine based severity adjustments, perform the following actions.

- Special K and Reduced K no longer apply for the parameter.

- Reduce the reference interval for the next scheduled reference test in the offending stand/engine by one candidate test, or the number of days equivalent to one candidate test.  This might be a percentage of the calibration period.

For a system with lab based severity adjustments, perform the following actions.

- Special K and Reduced K no longer apply for the parameter.

- Reduce the reference interval for the next scheduled reference test in the offending stand/engine by one candidate test, or the number of days equivalent to one candidate test.  This might be a percentage of the calibration period.

Issue 10:  Exceed Shewhart Stand/Engine Precision Action Alarm

For a system with stand or engine based severity adjustments, perform the following actions.

- Special K and Reduced K no longer apply for the parameter.

- Reduce the reference interval for the next scheduled reference test in the offending stand/engine by four candidate tests, or the number of days equivalent to four candidate tests.

For a system with lab based severity adjustments, perform the following actions.

- Special K and Reduced K no longer apply for the parameter.

- Reduce the reference interval for the next scheduled reference test in the offending stand/engine by four candidate tests, or the number of days equivalent to four candidate tests.

Issue 11:  Exceed EWMA Lab Severity Alarm

For a system with lab based severity adjustments, perform the following actions.

- Calculate and apply lab based severity adjustments to all candidates in the lab completed after the alarm after following rules for precision alarms.

For a system with stand or engine based severity adjustments, perform the following actions.

- None, but notify TMC.

Issue 12:  Exceed EWMA Stand/Engine Severity Alarm

For a system with stand or engine based severity adjustments, perform the following actions.

- Calculate and apply stand/engine based severity adjustments to all candidates in that particular stand/engine completed after the alarm after following rules for precision alarms.

For a system with lab based severity adjustments, perform the following actions.

- None, but notify TMC.

Issue 13:  Exceed Shewhart Severity Alarm

For a system with lab as the major source of variation (note that this is a subset of systems with lab based severity adjustments), perform the following actions.

- Immediately conduct an additional calibration test in the offending stand/engine.  However, if a severity adjustment existed in the lab prior to the reference test, and the alarm is in the direction of the severity adjustment, then an additional calibration test need not be run as long as the test result is within the Special K control chart limit.  However, if the Special K control chart limit is utilized, reduce the reference interval for the next scheduled reference test by two candidate tests, or the number of days equivalent to two candidate tests.

- If a test stand fails two consecutive calibration tests on the same parameter but on different reference oils, the stand must generate two (2) operationally valid calibration tests, with no Shewhart severity alarms (all parameters).

For a system with stand or engine based severity adjustments, perform the following actions.

- Immediately conduct an additional calibration test in the offending stand/engine.  However, if a severity adjustment existed in the stand/engine prior to the reference test, and the alarm is in the direction of the severity adjustment, then an additional calibration test need not be run as long as the test result is within the Special K control chart limit.  However, if the Special K control chart limit is utilized ,reduce the reference interval for the next scheduled reference test in that particular stand/engine by two candidate tests, or the number of days equivalent to two candidate tests.

Issue 14:  Operationally Invalid Reference Tests

For every 2 invalid reference tests in a calibration attempt, reduce the reference interval of the next available stand by one candidate, or the number of days equivalent to one candidate test, whether or not the original stand remains in the system.

Issue 15:  Setting K and Lambda Values

There is certainly a tradeoff in setting K values between detecting shifts and minimizing false alarms.  The larger the K, the smaller the false alarm error rate, and the longer the Average Run Length (ALR) of reference tests until an alarm.  While that is a positive, the downside is a smaller alarm rate and longer ARL when trying to detect shifts, trends and problems.  The error rates and Average Run Lengths for the Shewhart control charts are included in the ‘Probability of an Alarm’, and ‘Shewhart ARL’ tabs in Appendix A.  The plot of ARL as a function of K and the actual shift in severity is found in the ‘ARL Plot’ tab.  One can clearly see the tradeoff between false alarms and shift detection in this plot.  It may be wise to consider consequences of false alarms, consequences of not detecting shifts, and the number of reference tests that a lab or stand is expected to run in its lifetime when selecting a K value.

While calculating the Shewhart theoretical alarm error rates and the ARL are simple and straightforward using the normal distribution (assuming that the data is normal, that is), the calculations are a little bit more difficult for the EWMA because the calculations involve lambda and the data are correlated.  Therefore, simulation is used to calculate the ARL in the ‘EWMA ARL’ tab.  The concept of the higher the lambda, the more weight on the most recent data point in the EWMA control chart is not too difficult to understand, but setting lambda is not easy.  Given that the Shewhart charts are already an EWMA with lambda equal to one (1), we begin at 0.3 and below for lambda considerations.  Once again it may be wise to consider consequences of false alarms,

consequences of not detecting shifts, and the number of reference tests that a lab or stand is expected to run in its lifetime when selecting a lambda and K value for the EWMA charts.  For example, given the importance of precision, but the consequences of an EWMA precision alarm, plus the influence of outlier results on precision alarms (an outlier greatly affects two data points for monitoring precision) it may be a good idea to use a lower K (1.645, for example), and a lower lambda (0.1, for example).

Issue 16:  Template for Lambda and K

It is important to establish a general, standardized template for Lambda and K values from which to discuss any possible deviations for specific engine tests.  Suggestions for a starting point may be found in the ‘Generic Lambda and K’ tab in Appendix A.

Issue 17:  Deviations Based on Lab Size and/or Reference Frequency

While it appears that LTMS guidelines should be adjusted based on lab size and/or reference frequency, this is not necessarily the case.  There will always be inherent advantages and disadvantages to being a lab with many stands or being a lab with one stand.  If you are a lab with many stands, you are more likely to encounter a false alarm than a lab with a single stand.  However, the impact of a false alarm on overall lab economics is not as large.  If you are a lab with many stands, you are more likely to get that severity adjustment when the Industry is severe than a lab with one stand.  However, you are also more likely to get that severity adjustment when the Industry is mild.  Therefore, although there are easy to see advantages to being a large lab or a small lab, overall, the system is fair.  A large lab or a small lab may catch a bad break, and that is unfortunate, but the system is fair, and it would be unwise and detrimental to adjust the system for every perceived bad break.

In the past, we have been reluctant to use severity adjustments for tests with very low reference frequencies.  The point was that it may take several years to get 4 to 5 test results in order to generate a severity adjustment, and the relatively high weights on test results from years ago may not be relevant.  While this point is well taken, we may still use the same system for tests with a lower reference frequency that we do for a test with a higher reference test frequency.  Once again, there are tradeoffs, but the tradeoffs are fair.  A test with a higher reference test frequency is more likely to encounter a false alarm than a test with a lower reference test frequency.  However, the impact of a false alarm on a test with a higher reference test frequency is not as large.  A test with a higher reference test frequency is more likely to get those severity adjustments when the Industry is severe, but will also get those severity adjustments when the Industry is mild.

Large or small labs, high or low reference frequency, we have said that the system is fair.  However, there are points at which the system will not even work approximately well.  It may be argued that there are several test types currently in LTMS that fit this description, but it will not be argued here.  The point is that if a test generates results that are not approximately normally distributed, are not well behaved, cannot be well characterized beyond descriptions like ‘fail-safe’, exhibit test apparatus by oil interactions, and rarely see multiple repeats on batches of parts and/or fuel, then that test should not monitored using LTMS, and candidate oils in the test should not be subject to registration or Multiple Test Evaluation Procedures.

Issue 18:  Rule of 8’s

Consider modifications to IIIG LTMS that deals with trigger value for application of severity adjustments.  Continue with the “normal” application of severity adjustment following the exceeding of the Zi threshold value.  If the SP believes the test is in a severe or mild state and a lab has 5 (or some SP determined) number of Yi data in a row in the same direction of shift, reduce the K value for application of severity to 0 for that parameter until such time as the 5 in a row number is no longer satisfied or the SP deems the test parameter is no longer in the severe or mild state.

Summary

While each test type and Surveillance Panel is unique with different needs, it appears that structure, logic, and purpose have been absent in setting up LTMS for new test types.  Therefore it is suggested, in this document, to standardize LTMS across test types and provide a consistent template for all test types.  LTMS guidelines are suggested in order to provide the best possible basis for level-playing-field, unbiased, cost-effective candidate testing.  Will use of these guidelines ever produce a false alarm?  Yes.  Will use of these guidelines ever fail to capture a true shift in severity?  Yes.  Will the guidelines ever favor a lab with many stands?  Yes.  Will the guidelines ever favor a lab with one stand?  Yes.  However, the guidelines still do represent the best possible basis for level-playing-field, unbiased, cost-effective candidate testing.

Appendix A

Appendix B

For additional history and information on LTMS, please read SAE Paper 2004_01_1891.