Proposed Procedure Revision:

9. Engine/Stand Calibration and Non-Reference Oil Tests

9.1 General – Calibrate the test stand by conducting a test with a blind reference oil16.  Submit the results to the ASTM Test Monitoring Center (TMC) for determination of acceptance according to the Lubricant Test Monitoring System (LTMS)16.

9.2	New Test Stand — A new test stand is defined as a test stand that has never been calibrated or has not completed an acceptable reference oil test within 18  XX months of the end of test (EOT) date of the last acceptable reference oil test.  Under special circumstances, such as industry-wide parts or fuel shortages, the TMC may extend the time period beyond 18 XX months.  Perform the following to introduce a new test stand.

9.2.1 New Test Stand Calibration – New stand calibration is determined according to the LTMS16.

9.3	Stand Calibration Period – The calibration period is 12 24 months from the EOT date of the last acceptable reference oil test or the completion of 10 500 hour non-reference oil tests, whichever comes first.

    - It is recognized that an idle test stand can be subject to conditions that can cause performance shifts; therefore if the idle period between valid tests exceeds 60 days, a full instrument calibration must be performed before the next test is started.  If the idle period exceeds 120 days, the full instrument calibration must be performed and a workable slave engine mounted in the test stand and run for 20 hours under test conditions to demonstrate correct operation of all control systems.  These remedial actions must be reported on Form 11 of the test report.

9.3.1	The TMC may schedule more frequent reference oil tests or extend the calibration period.