Stage 2 Operational Conditions

| Step | Cycle Timer | Accumulated Time | Operation | NGET Throttle Position % | Speed (RPM) | Load (Nm) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | - | - | Idle | 0 | 800  25 | No load excitation |
| 1 | 2.5 | 2.5 | Accelerate to step 2 conditions | 100 | Ramp up | Ramp up |
| 2 | 6.0 | 8.5 | Rated speed | 100 | 2,600* | WOT(800 nominal) |
| 3 | 2.0 | 10.5 | Lug** to step 4 conditions | 0 | Ramp down | Ramp down |
| 4 | 1.0 | 11.5 | Idle | 0 | 800  25 | No load excitation |
| 5 | 2.5 | 14.0 | Accelerate to speed & load | 75*** | 1,600 minimum | 550 – 750 Nm |
| 6 | 2.0 | 16.0 | Lug** to step 7 conditions | 0 | Ramp down | Ramp down |
| 7 | 1.0 | 17.0 | Low Idle | 0 | 800  25 | No load excitation |
| 8 | 2.5 | 19.5 | Accelerate to speed & load | 75*** | 1,600 minimum | 550 - 750 Nm |
| 9 | 2.0 | 21.5 | Lug** to step 10 conditions | 0 | Ramp down | Ramp down |
| 10 | 1.0 | 22.5 | Idle | 0 | 800  25 | No load excitation |
| 11 | 2.5 | 25.0 | Accelerate to speed & load | 75*** | 1,600 minimum | 550 - 750 Nm |
| 12 | 2.0 | 27.0 | Lug** to step 13 conditions | 0 | Ramp down | Ramp down |
| 13 | 1.0 | 28.0 | Idle | 0 | 800  25 | No load excitation |

*Engine speed average (6 seconds) to be 2,600 +/- 50 RPM.

**Engine speed must come to idle before reaching a no load condition

***Approximate throttle position to reach specified speed and load.