ISM O&H Panel Teleconference

January 8, 2004

# Attendance

## Cummins

Warren Totten

## Ethyl

Bob Campbell

## Lubrizol

Dan Domonkos

Bill Larch

## PE

Jim Moritz

John Hagelin

## SwRI

Scott Richards

Andy Broff

## Infineum

Jim Gutzwiller

## TMC

Jeff Clark

# Call to Order

Warren Totten called the meeting to order at 2:02 PM Eastern Time.  The call ended at 2:45 PM EST.

# ISM Stand Update

Ethyl – no change

Lubrizol – Communication issues have plagued efforts to generate data using the NGET/Inline II adapter.  A new computer will be used and efforts to troubleshoot will continue.  The M11 EGR lube filter head needs to be changed also.  Once the communication issues are resolved mapping data based on the two M11 EGR stages will be generated and submitted to the panel.

PerkinElmer – M11 EGR filter head is being retrofitted.  Data has been generated; however, there are power output differences from the stand at SwRI.  Jim would like the Stage II timing issues to be addressed, EGR limits, and determine discrepancies between the engine stands.  In addition, they may need to swap out the dyno.

Southwest Research – Oil cooler and oil filter head retrofit have been completed.  Photos will be sent out to the O&H panel.  Scott would like to get soot map data, determine the difference between PE and SwRI.  Currently there is a 40 HP difference between the two stands.  SwRI is running 30 kPa higher boost.  Intake CO2 levels are .77 vs. .6 respectively and turbine speed is 103KRPM vs. 109KRPM.  Scott would like to see Lubrizol data ASAP for comparison.

# Discussion

The labs discussed how the timing should be changed.  The question arose concerning what to do during shut down and start up.  Cummins suggested shutting the 12VDC power off to the Inline II to sever communications during shut down and start up.  The labs agreed that this method would work.  Additional discussions around timing tables came to a close as Cummins stated that there were no tables as in the old M11 EGR/HST tests and that the timing is calculated real time.  Therefore, the only option is to change the timing at stages A and B.  The group will need to decide what timing should be run during Stage B.  All agree that it is in the 4 to 4.5 range.

# Action Items for 12/18/2003

Cummins Inc.

Review service literature for new ISM shop manual (11/13)

Provide NGET training on a one-on-one basis with each lab (as needed)

Facilitate shipment of “excellent” oil to TMC (currently in process)

Return re-flashed Inline II adapters to TEI (1/8)

Follow-up on block delivery (1/8)

Set time and date for face to face meeting in San Antonio (1/8)

Labs

Order and prepare parts rebuild kits for matrix run (pre-measurements)

SwRI to send photos of oil filter head and cooler to O&H panel

# Next Meeting

The next meeting is Thursday, January 15, 2004.  The teleconference number is 812-377-6156.