# Cummins ISM Lubricant Test Procedure

- Scope

This document procedure covers the Cummins ISM Lubricant Test which was developed to insure backwards compatibility of engine oils formulated to function in model year 2007 engines and beyond and in pre-2007 engines.

- Reference Documents

## Hardware and Modification Requirements

- The ISM test platform will be set-up as in a M11 EGR test stand

- Rebuild Engine with ISM  Lubricant Test Rebuild Kit.  (Attachment 1)

- The oil cooler needs to be reversed like for the M11 HST but the cooling water needs to provided from other than the engine.

## Operating Locations and Hardware for ISM as specified for the M11 EGR.

- Fuel Supply

- Coolant System

- Pressurized Oil Fill System

- External Oil System

- Blowby  Rate

- Intake Air System

- Engine Coolant

- Coolant Out Temperature

- Coolant In Temperature

- Fuel In Temperature

- Oil Gallery Temperature

- Exhaust Temperature

- Oil Gallery Pressure

- Oil Filter Inlet Pressure

- Oil Filter Outlet Pressure

- Crankcase Pressure

- Air Intake Pressure

- Exhaust Pressure

- Fuel Pressure

## Pre - Test Measurements

- Crossheads – weight (same as M11 HST/EGR)

- Rocker Swivel Foot – remove from rocker lever, clean, and weigh per crossheads and replace – DO NOT APPLY LUBRICANT AT ANY TIME PRIOR TO REBULD

- Injector Adjusting Screws –clean adjusting screws same as crossheads, weigh individual screws.

- Rod Bearings – Weight (same as M11 HST/EGR)

- Rings – Weight (same as M11 EGR)

- Cylinder Liner Trace

## Test Procedure

- Install standard LF9009 filter

- Fill engine with 54.6 lbs. oil – Same as M11 EGR

- Fuel must meet specifications (per Attachment 2)

- Run Warm Up Procedure.  (Table 1)

- Run Break-in  Procedure

- Take engine to condition B for two hours.  (Table 2)

- Verify cylinder pressure meets operating goals.

- Drain oil, change filter to double wire mesh oil filter and fill with 54.6 lbs. of test oil.

- Run Condition A (high soot portion of test) (Table 2)

- Adjust timing (TVC) as required to obtain necessary soot rate.

- At end of 50 hours,

- Verify soot is within specifications.  (Table 2)

- Run Condition B for 50 hours (EGR portion of test).

- Control speed.

- Control fuel rate.

- Control oil temperature.

- Control intake manifold temperature.

- Control water outlet temperature.

10.  Every 8 hours.

- Check EGR Rate at Condition B

- If average EGR rate of two 8-hour periods exceeds the specifications then adjust the EGR rate and record.

- 11.  Every 25 hours.

- Take oil sample every 25 hours.

- Force oil add 1.4 KG.

12. EOT – Same as M11 HST

- Oil drain same as M11 HST.

- Valve cover and oil pan removal same as M11 HST.

## Post Test Measurements

- Oil Pan & Valve Cover -  Sludge Deposits – Same as M11 HST

- Crossheads – Weight loss – Same as M11 HST

- Rocker Swivel Foot – Weight loss – Same as crossheads

- Top Piston Ring – weight loss

- Oil Analyses – Table 3

- Piston Deposits – Same as 1P

- Cylinder Liner – Step Wear Measurements as per Modified T-9 Liner Wear Procedure.

- Adjusting Screw –Weight loss

- Push tube – no scuffing, either end

Table 1: Warm Up Conditions

| Parameter/Stage | Unit | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- | --- |
| Stage Length | Min | 5 | 5 | 5 | 5 | 5 |
| Engine Speed | R/min | 700 | 1200 | 1600 | 1600 | 1600 |
| Engine Torque | NM | 135 | 270 | 540 | 800 | 1084 |
| Intake Manifold Temperature | C | 70 | 70 | 70 | 70 | 70 |
| Oil Gallery Temperature | C | 130 | 130 | 130 | 130 | 130 |
| Coolant Out Temperature | C | 105 | 105 | 105 | 105 | 105 |

**Maximum temperatures shown

Table 2: 200-h Test Operating Conditions

| Parameter/Stage | Unit | A (Soot) | B (Wear) |
| --- | --- | --- | --- |
| Stage Length | hr | 50 | 50 |
| Engine Speed a | r/min | 1800  5 | 1600  5 |
| Torque ab | Nm (lbft) | 960 | 1424 |
| Fuel Rate | kg/hr (lb/hr) | 128 + 2 | 144 + 2 |
| Intake Manifold Air Temperature | oC (F) | 150 | 150 |
| Turbo Inlet Air Temperature | oC (F) | Record | Record |
| Barometer | kPa (inHg) | Record | Record |
| Humidity (Dew Point) |  | Record | Record |
| Oil Gallery Temperature | oC (F) | 240 | 240 |
| Coolant Out Temperature | oC (F) | 150 | 150 |
| Coolant In Temperature | oC (F) | Record | Record |
| Coolant Delta Temperature | C (oF) | 7 max | 7 max |
| Fuel Temperature | C (oF) | 40  2 | 40  2 |
| Inlet Air Pressure | kPa abs | Record | Record |
| Exhaust Back Pressure | kPa abs | 107  1 | 107  1 |
| Intake Manifold Pressure | kPa abs | Record | Record |
| Coolant System Pressure c | kPa | 99 - 107 | 99 - 107 |
| Crankcase Pressure | kPa | Record | Record |
| Oil Gallery Pressure | kPa | Record | Record |
| Oil Filter Delta Pressure | kPa | Record | Record |
| Blowby | L/min (inH2O) | Record | Record |
| TVO | Degrees | 307  3 | 347 |
| TVC | Degrees | 16 min | 32 |
| Soot Rate | % | 3.0  0.20d e |  |
| Turbo Speed | r/min | <115,000 | < 115,000 |
| Cylinder Pressure f | Kpa | N/A | 18000 - 19500 |
| EGR Rate (during break-in) | % | N/A | 9.0-9.8 |
| EGR Rate (during daily checks) | % | Record | 8.5-9.8 |
| Intake Manifold CO2 (break-in) | % | Record |  |
| Intake Manifold CO2 (daily checks) | % | Record |  |

a Values are for reference purposes

b At standard atmospheric temperature and pressure.

c  Coolant system pressure is measured on the top of the expansion tank.

d  Minimum soot at 50 hours is 2.8%

e Target soot rate at 150 hours is 5.0%

f  Cylinder pressure measurement is taken on cylinder number 6

Table 3: Oil Analysis Requirements

| Sample Hour | Metals | TAN D664 | TBN D4739 | Vis @ 100 oC CG-4 | Soot TGA, CG-4 |
| --- | --- | --- | --- | --- | --- |
| 0 | X | X | X | X | X |
| 25 | X |  |  | X | X |
| 50 | X | X | X | X | X |
| 75 | X |  |  | X | X |
| 100 | X | X | X | X | X |
| 125 | X | X | X | X | X |
| 150 | X | X | X | X | X |
| 175 | X | X | X | X | X |
| 200 | X | X | X | X | X |
| 225 | X | X | X | X | X |
| 250 | X | X | X | X | X |
| 275 | X | X | X | X | X |
| 300 | X | X | X | X | X |

Attachment 1

Description	Cummins P/N		Per Kit	Critical

Cam Follower Parts

Injector cam follower assy	3417645		6	Y

Valve cam follower 	3161475		12	Y

Cam follower shaft	3417766		2	Y

Cam follower shaft support (end)	3064583		1	Y

Cam follower shaft support	3895831		1	Y

Cam follower shaft support (center)	3895830		5	Y

Cam follower shaft support (end)	3064582		1	Y

Plain washer	3009330		7

Cylinder Head Parts

Expansion plug	3895479		2	Y

Expansion plug	206741		2

Expansion plug	3007635		8

Valve seat (intake/exhaust)	3090704		24

Valve stem guide	3328786		24

Valve spring	3895860		24	Y

Intake valve	3417778		12

Valve collet	3275354		48

Exhaust valve	3417779		12

Valve spring retainer	3883512		24	Y

Spring guide	3070072		24

Pipe plug	3008465		16

Orifice plug	3820749		6

Valve stem seal, intake	4003966		12	Y

Valve stem seal, exhaust	4003966		12	Y

Overhead Components

Rocker lever assembly (exhaust)	4003914		3	Y

Rocker lever assembly (exhaust)	4003911		3	Y

Rocker lever assembly (intake)	4003913		3	Y

Rocker lever assembly (intake)	4003912		3	Y

Rocker lever assembly (injector)	3069020		6	Y

Rocker lever shaft	3417765		2	Y

Valve crosshead (EGR batch)	3070175		12	Y

Push rod	3068390		12

Push rod	3076046		6

Rocker lever support	3893584		4

Rocker lever support	3079662		2

Rocker lever support	3079661		2

Retaining clamp	3077444		8

Pistons/Rings/Liner

Piston top	3896030		6	Y

Piston skirt	4070653		6	Y

Piston pin	3063843		6

Retaining ring	3016652		12

Top compression ring	3102424		6	Y

Second compression ring	3899413		6	Y

Oil ring	3102506		6

Cylinder liner	3102423		6	Y

Miscellaneous Components

Pressure regulator plunger	3068979		1

Compression spring	3010146		1

Retainer plug	3895718		1

Piston cooling nozzle	3080708		6

Rectangular seal	3047188		6

Oil cooler	3081359		1

Connecting rod bearing	3016760		12	Y

EGR Hose	3100062		2	Y

Oil filter	EGR ASTM 3	                      1	      Y

Attachment 2

Phillips RD-9 (task force may wish to re-bid fuel for the ISM)