Sample T-12 EGR Cooler Cleaning Method

NOTE: This method is not mandatory; it is simply an example of a cleaning method that has allowed T-12 EGR coolers to successfully be reused.

- Flow compressed air backwards through the coolers.

- Flow hot water backwards through the coolers.

- Fill the coolers with a hot water and Tide detergent solution and submerged them in bucket of hot water. Periodically, move or agitate the coolers.  Repeat this step.

- Flow hot water through the coolers until they run clean.

- Turn the coolers upside down and blow compressed air through them. Then let the coolers sit upside down until they were dry.