RFWT REFERENCE TESTING SUMMARY

Prepared for the

September 1998 Conference Call

Two RFWT tests, from one lab, have been completed during the October 1998 report period which began April 1, 1998 and ends on September 30, 1998.  The following summarizes these tests:

|  | Number of Tests |
| --- | --- |
| Operationally and Statistically Valid | 2 |
| Failed Acceptance Criteria | 0 |
| Not for Industry Statistics | 0 |
| Non-blind, Aborted | 0 |
| Non-blind, Information | 0 |
| Total | 2 |

Attached is the current industry severity and precision EWMA control charts for Average Follower Wear (AFW). The industry is currently within the control chart limits for AFW.

Current reference oil test targets for Average Follower Wear are shown below:

| Oil | n | Mean (mils) | Std. Dev. |
| --- | --- | --- | --- |
| 1004-2 | 10 | 0.33 | 0.05 |
| 1004-3 | - | 0.33 | 0.05 |
| 1005 | 6 | 0.20 | 0.06* |
| 1005-1 | - | 0.20 | 0.06* |

*Based upon historical test precision.

Targets for oils 1004-3 and 1005-1 are based upon the targets for oils 1004-2 and 1005 respectively.  To date, 2 tests have been run on oil 1005-1 and no tests have been run on 1004-3. Note, that the current pooled standard deviation for oils 1005 and 1005-1 is 0.03.  This value is half the current target value.  This does not represent an improvement is test precision so much as it represents an improvement in reference oil performance compared to oil 1004.  This also raises the question of which standard deviation should be used for Severity Adjustment calculations.  Current SA calculations are based upon oil 1004 (and reblends).

Surveillance Panel Action Items:

1. Proposed Motion Regarding Reference Oil Test Targets:

Targets for both 1004-3 and 1005-1 to be updated at five tests.

Targets for 1005 to be updated at ten tests.

Targets for 1004-2 to be updated at twenty tests.

2. Address status of discrimination tests and the use of reference oil 1001.

3. Address SA calculations.

A complete semi-annual report for the Roller Follower Wear Test will be published in early October.