ESCIT Notes 7/14/07

ACC Headquarters

- Scott Ellis shows data on in-house catalyst test

  - Shows data on Afton oil (zero Phos)

    - 3.1L GM engine  Special research catalysts.

      - Can put in other catalysts with cutting and re-canning.

    - Murray indicates that this is the type of test the OEMs have wanted for years.  Test could be useful for future engine oil work.

- ROBO

  - Kinker admits there is very little correlated data

  - ROBO Phos retention appears to be a little higher and outside IIIG EOT control limits.  Not as severe.

  - 8.5% separation between 834 and 835

  - Some repeatability concern.  10% difference for oil B

  - Too much work and too little time

  - Concluded drop PROBO from ROBO

- Infineum

  - Supports IIIG@20

    - Adequate precision

    - Relevant to field experience

  - Also support PEI165

  - Do not support other tests

  - IIIG@20 shows 75% PR +/- 10%.

    - They also ran flush and run.  Found similar results

  - Running PEI165-16

    - Typically get results from 40-60 mg at Intertek  Similar to 434

  - PEI165 delta about 10 mg for two oils.  Difference across labs is 20 mg (Intertek and Savant)

- LZ

    - Shows high idle NYC taxi fleet.  20 hour IIIG 20% active ZDP.  43% at 1019 miles and 18% at 1813 miles

    - Snider asks if we can get further data on active ZDP in field trials

      - Snider asks LZ thoughts on 34% active ZDP in IIIG@20.

      - GM has a concern with 20 hour data.  Feel all ZDP is not volatilized until 40%

- Afton

    - Can live with either test but support PDI165

    - Afton only runs tests at Savant.  Ran about 70 tests

      - Calculates pooled Std Dev at 1.97 mg.  Five repeats 1.87 mg

    - Ran Afton Catalyst test.  Looting at average increase in T50.  Been running ~2 years.

      - Tries to show correlates with PEI

      - Operating conditions similar to IIIG

        - Looks at drain retention in catalyst test.

    - Afton plans to provide a SAE paper on Afton Catalyst test this fall

    - Guinther concerns with IIIG

      - Costly

      - Testing resources

      - Concern with ICP

        - Manufactures data to show up to 9.5% variability

        - Need to make ICP a monitored measurement

        - Sees 6.7% variability in his Afton Cat

      - Doesn’t like flush and run IIIGPR20

    - Afton Combined-parameter emission system improvement proposal

      - Romano pushes OEM desire for 0.05% equivalent system.   Present weighting system doesn’t’ lean that way.

- Selby (Savant)

    - Proposes a round robin with analysis complete in end Sep.  ASTM procedure in Oct.

- GM concerned with full volatilization of Phos.  Extend both PEI and IIIG@40

    - Selby thinks bad idea for round robin.

    - Shows curve with PEI run out to 48 hours.  Oils that looked very different at 16 hours (around 17 and 30, but both were around 95-100 at 48 hours.

      - 16 hour duration is so you can run it overnight

      - Changing time is a R&D issue - Selby

      - Oils are “atypical”.  Common in market place.  Oil 1 SAE 10W-30 (GF-3, SL) and 10W-30 (API SJ, CH-4)  Showed a lot of volatility at 250C

      - GM uncomfortable with the convenience aspect of PEI-16.  Would like to see a matrix

      - No other data on oils out to 48 hours

        - AAM willing to help fund to answer question of test duration

        - Murray asks how long it will take?  It’s research….

        - Romano says we already have correlation at 16 hours

          - Linden pushes back.  “I’d rather have a test that takes longer if it gives me what I need”

          - Murray concerned about time to research

          - Williams – let’s get out procedure and make sure all run the same.  Matrix separate.

          - LZ points out steep slope at 16 hours.  McMillan – lots of things still changing

          - Savant has 4 stands

          - Agree to task force

            - Infineum

            - LZ

            - Ford

            - GM

            - Afton

            - DCX

            - Intertek

            - Oronite

            - Need statisticians

            - Would be nice if IIIG results available

    - GM references Oronite data suggesting bottom occurs at 40 hours

      - GM doesn’t think need to go beyond 40 hours

      - Oronite argues complexity

        - Commercial data not usually submitted unless blind

        - Says difference between 20 and 40 hours are smoothed averaged data = 3%

      - LZ offers to go back and look at 20 to 40 hour to find delta and see if data is flat

      - Linden asks ACC to contribute data

- Murray presents timeline

  - Early Aug – Feast of data

  - Informal ballot

  - Meeting in Sept to review

  - Recommendation to ILSAC/OIL in Oct

  - Presentation to ASTM in Dec

  - Winter 08 ASTM Panels

  - March ASTM ballot and ESCIT dissolution

- Next meeting 9Aug07.  Midland (Savant)