January-February 1996 (Precision Alarm)

The industry experienced a precision alarm of six data points.  This alarm was caused by three reference oil tests at one laboratory. The data from this laboratory, which was trending mild, was intermixed with data from two laboratories with on target performance.  No other cause for this alarm was found and the alarm cleared with subsequent testing at other laboratories.

April 1996 (Precision Alarm)

The industry experienced a single point precision alarm. This alarm was due to two laboratories with opposite severity levels (one mild, one severe) reporting two reference oil tests in alternating order. This was sufficient to return the industry to an alarm condition since the industry was still close to the limit from the January-February 1996 alarm event. No other cause for this alarm was identified and the alarm cleared with subsequent testing.

August 1996 (Precision Alarm)

The industry experienced a single point precision alarm. This alarm was due to two laboratories with opposite severity levels (one mild, one severe) reporting three reference oil tests in alternating order. No other cause for this alarm was identified and the alarm cleared with subsequent testing.

October 1996 (Severity Alarm, severe direction)

The industry experienced a single point severity alarm. This alarm was due a single test laboratory, which was trending severe, reporting seven of the eight data points leading up to the alarm. No other cause for this alarm was identified and the alarm cleared with subsequent testing from other laboratories.

August 1998 (Severity and Precision Alarms, severe direction)

The industry experienced severity and precision alarms of two and three data points respectively.  These alarms were caused by two reference oil tests at a single test lab, both on reference oil 402-1 (these same two tests caused the precision alarm on HRS).  No cause for the alarm has been found and the laboratory has generated passing results on a subsequent reference oil test. Subsequent testing has cleared the alarm condition.

December 1998 (Precision Alarm)

The industry experienced a single point precision alarm. This alarm was caused by a single preceding test on reference oil 402-1 that failed severe on AES (-3.04 /s). No other cause for this alarm was identified and the alarm cleared with subsequent testing.

March 1999 (Severity & Precision Alarms, severe direction)

The industry experienced severity and precision alarms of two and five data points respectively.  These alarms were caused by two reference oil tests at a single test lab, one on reference oil 402-1 which failed severe on AES (-2.83 /s) and one on reference oil 403 which generated severe but passing AES results.  No cause for the alarm has been found and the laboratory has generated passing results on a subsequent reference oil test. Subsequent testing has cleared the alarm condition.

The industry has experienced no other alarms.