# SEQUENCE VG ROCKER COVER SLUDGE INDUSTRY ALARM LOG

March 23, 2000 to June 19, 2000 (Severity, Severe Direction)

Alarm Event begins with a result –5.267 /s from target, followed by results –0.800 and –1.633 /s from target. Results were all on 1007 and from two stands in the same lab. Alarm event continues with the reporting of a result –0.9 /s from target using reference oil 1007. Test remains in action alarm when a result completing on 4/30/00 is reported, -1.462 /s from target using reference oil 1006 on an existing stand. Alarms continue with the bulk of the tests reported one standard deviation or more from target. The latest two results were from a new stand. The stand ran both 1006 and 1007 and were consistently severe by 1.7 standard deviations. Alarm finally clears after 2 results are reported from a different lab, 0.9667 and 1.000 Δ/s from  target.

March 23, 2000 to May 8, 2000 (Precision)

Precision alarm sounds the same time as the severity alarm and appears to be caused by the same results. Precision remains in action alarm on 4/18/00 when result –0.900 /s from target is reported after a result –1.633 /s from target has been reported on 4/8/00. Both tests were on reference oil 1007 from different labs. Test remains in action alarm when a result completing on 4/30/00 is reported, -1.462 /s from target using reference oil 1006 on an existing stand. Alarm finally clears on May 8, 2000. It should be noted that most results reported since April 30 have been about –1 standard deviation from target.

January 20, 2001 to March 13. 2001 (Severity, Severe Direction)

A series of three severity action alarms have occurred caused by test which were –2.639, -4.220, -1.611 and –1.244 Δ/s from target. The –2.639 and –1.611 results were from the same stand on reference oil 925-3. The remaining tests were both on reference oil 1007 and were from different labs and different stands. Three of the four results were from the same lab. Alarm goes to warning level when result close to target is reported. Failing result on reference oil 1006 cause alarm to go back to action level.  Three results from the same lab, which were on target to slightly mild cause the alarm to clear.

February 18, 2001 to Present (Precision)

Precision warning alarm begins on February 18, whne a result –2.75 Δ/s from target is reported which was proceeded by a result 1.194 Δ/s from target. Alarm goes to action level on March 9 when a result 0.244 Δ/s from target is reported, and returns to warning level on March 13, when a result 0.75 Δ/s from target is reported. The precision alarms appears to be the result of the test which was –2.75 Δ/s from target, reported between a series of 4 slightly mild results. These 5 results were reported from 2 labs.

March 20, 2001